# ANNUAL CONDITION INSPECTION

REMOVE ALL INTERIOR AND EXTERIOR INSPECTION COVERS, COWLING, WHEEL PANTS, AND SPINNER

GENERAL GROUP

- Add Squawks and Might–As–Well items

- Review Velocity Aircraft website for “Kit Plans Changes” (KPCs)

- Check for ADs at the Airworthiness Directives Biweekly website applicable to certified components/ accessories

- Review items on Squawk List

- Review W&B and Equipment List for currency. Reweigh at the first annual and every 3 years thereafter or after any major modification

## LIGHTS – Check:

- Navigation

- Strobes

- Landing

- Panel Dimmer

- Reading Lights

## CANARD GROUP / NOSE COMPARTMENT

- Nose Access Cover – Operation and damage

- Canard Cover – Hinge attachment and condition

- Canard Attachment Bolts – Condition and torque

- Canard -- Damage, delamination and cracks

- Elevator Travel –28° down/24°-26° up

- Elevator Hinges –  Screws, operation and damage

- Elevator Counterweights – Covers, attachment  and damage

- Pitch Trim Motor System – Security, operation and range

- Autopilot Pitch Servo System – Security, operation and range

- Elevator/Control Stick – Bolt and connecting rods

- Elevator Torque Tubes / Center Hinge – Attachment and damage

- Nose Gear Strut – Attachment and damage

- Rubber Bumper System – Security

- Oil Lines – Connections, leaks, security

- Fresh Air intake – Hose condition, attachment and security

- Batteries/Bus Fuses – Security, condition and connections

- Contactors – Component attachment and wires

- Electrical System – Component attachment and wires

- Pitot tube System – Connections, leaks, heating

- Voltage Regulator – Attachment and condition

- Brake System – Security, operation, fluid level and leaks

- Pedals – Attachment and operation

- Oil Cooler – Attachment, damage, obstructions and leaks

- Heating System – Hose condition and operation

- Nose Tie down – Condition

- Landing Light – Condition and wiring

- Panel Components – Security, condition and connections

FUSELAGE

- Landing Light Lens – Condition and security

- Nose NACAs – Condition

- Static Air Ports – Obstructions

- Windshield – Condition

- Speed Brake – Condition, hinges, motor, and operation

- Doors
Hinge screws & pins
Lubricate door locking mechanism.
Check cotter pins
Check annunciator lights operation

- Windows – Condition

- Strakes Surface/ Passenger Windows / Tie Downs/ Fuel Caps – Condition, security and operation

- Fuel Leak Stains – Surface

- Overhead NACAs – Condition, obstructions

- Upper Cowling – Condition, oil access door for security/operation

- Bottom Cowling – Condition

- Cowling Interior – Flanges, hinges, pins

- Transponder Antenna – Condition and security

- Fuel Vent – Obstructions

- Sump Tank Drain – Condition and operation

- Fuel Pump/Induction Fuel Drains – Condition and obstructions

- Cowl Hinges – Hinge attachment and condition

# PROPELLER GROUP

- Propeller/ Spinner – Damage, delamination, cracks and security

- Propeller Hub – Bolts (65 ft–lbs dry torque) safety wire

- Blade End Play – < 1/8”

- Blade Travel – < 2°

- Blade Tracking - < 3/16”

# ENGINE GROUP

- Cowl Exterior and Interior – Damage, delamination and cracks

- General – Loose bolts, nuts or clamps in engine compartment

- Cooling Plenum – Remove.  Condition, leaks, cracks, baffles

- Wash Engine – Check for oil leaks

- Oil / Filter Change – If due, every 50 hrs

Open oil filter and inspect for contaminants and send for analysis

- Oil Temperature Sensor/System – Leaks and security

- Rear Oil Cooler – Leaks and security

- Spark Plugs – Condition. Clean, gap and reinstall with anti–seize.

Gap tool has .015 & .019 on one end

Aircraft Plugs – .015 must pass and .019 must not

Auto Plugs -- .30 must pass and .35 must not

Torque Plugs:  30 ft/lbs

- Cylinder Compression
#1          /80     #3          /80     #5          /80
#2          /80     #4          /80     #6          /80

- Cylinders – Broken or cracked fins

- Valve Covers – Oil leaks, security, torque 20–in. lbs

- Push Tube Seals – Oil leaks

- Ignition Harnesses – Condition

- Slick Magneto – 20° BTDC. Magneto points for gap and condition, oil leaks, magneto switch and “P” leads for operation.

- Air Induction System – Condition and leaks

- Air Filter – Clean, inspect, recharge and reinstall

- Air Oil Separator – Clean and reinstall

- Crankcase Vent Check Valve – Remove, decoke, clean, check operation, reinstall

- Fuel Filter – Clean and reinstall

- Fuel Injectors – Leaks

- Fuel Divider And Lines – Leaks, security

- Intake Tubes/Seals – Leaks, condition

- Flexible Fuel Lines – Leaks, condition

- Fuel Control/Injector – Leaks, security

- Fuel Vent System – Condition, security

- Fuel Drain – Condition and leaks

- Electric Fuel Pump – Turn on look for leaks and pressure reading

- Exhaust Stacks – Connection, gaskets

- Engine Controls – Condition, security, travel, rigging, lubricate

- Engine Mount – Cracks, security, bolt condition, washers, shims and torque

- Alternator – Condition, security, wires, belt tension

- Rudder Cables – Return springs, condition, lubricate

- Aileron System – Bell cranks, cables and rod ends. Condition and security.  Lubricate as needed

- Starter – Security.  Clean service shaft with silicone oil

- Wiring – Condition, security

# CABIN GROUP

- Fuses – Condition

- Door Latches – Operation and condition

- Door Locking Pins / Receptacles – Clean and lubricate

- Open Door Annunciators – Operation and condition

- Upholstery – Tears condition, operation

- Seat Belts – Tears, fraying, security

- Seats – Check bolts

- ELT – Operation, security

- ELT Switch battery, replace every 5 years (2017)

Duracell PX28L 6 volt Lithium battery, or equivalent

- Audio Alert Indicator, replace every 10 years (2022)

Duracell CR–2, 3 volt or equivalent

- ELT Primary Lithium, The lithium battery (P/N E–04.0) must be replaced on, or before the battery expiration date marked on the battery

- AP Roll Servo – Condition, operation, lubricate as needed

- Ailerons System – Condition, operation, lubricate as needed

- Roll Trim – Condition, operation

- Sump Tank – Attachment and leaks

- Transponder Antenna – Condition and connection

- Fuel Shutoff Valve – Condition, operation and leaks

- Brake Lines (Rear) – Condition, security

- Electrical/Electronics – Condition, security, operation

- Wiring – Condition, security

- Fuel Vent System – Condition, security, operation

- Rudder Pedals – Operation, security, lubricate as needed

- Brake Lines (Front) – Condition, security

- Rudder Controls – Condition, security, operation

- Pitot/Static System – Plumbing, condition

- Cabin Heater Control / Ducting – Condition, operation

- Control Stick – Wiring, condition, operation

- Speed Brake Actuator – Wiring, condition, operation

- Cabin Air Vents / Cabin Lights – Condition, operation

- Fuel Shutoff Knob – Condition, operation

- Panel Instruments – Condition, security, operation

- Radios Avionics – Condition, security, operation

- Compass – Calibrate if needed

- Rear Cabin Panels – Reinstall

# WING GROUP

- Wing surfaces for delamination, cracks and condition

- Wing vortilons for condition and security

- Fuel caps for condition and operation.  Clean, lube and reinstall

- Fuel tanks for leaks, including fuselage

- Winglet surfaces for delamination, cracks and condition

- Rudders for condition, travel (3 ½” – 4”) and security

- Rudder hinges, return springs.  Lubricate if needed

- Rudder control horns / cables for condition and security

- Rudder operation

- Ailerons for condition, travel (2-1/2” up and down), cracks and security

- Aileron hinges for condition and operation. Lubricate if necessary

- Aileron bell cranks, cables, and rod ends for condition. Lubricate as needed

- Aileron torque tubes and attachment points for condition

- Aileron bearings / brackets for condition and operation

- Wing attach bolts for condition and security

- Wing fairings and inspection plates for attachment and condition

- Polish and wax strobes

# LANDING GEAR GROUP

- Wheel pants for cracks, cuts and condition

- Nose gear strut for condition and security

- Shimmy damper system for condition, adjustment and security

- Brake pads for condition and wear  .100 minimum thick

- Brake discs for pitting and wear.  .327 minimum thick

- Brake calipers for condition and leaks

- Brake lines Leaks

- Bleed brakes – Fill fluid to reservoir

- Heat Shields – Condition, reflective insulation

- Tires for wear and condition

- Tire pressure		 Main: 55psi		 Nose: 45psi

- Main wheel bearings.  Clean and repack

- Main gear legs for integrity, cracks

- Main gear axles and system for condition and attachment

- Main gear camber – axle 10° down with plane on jacks and level

- Main gear toe in 1” per side

# OPERATIONAL GROUP

- Fuel pump and shutoff

- Speed brake operation

- Oil pressure/temperature indication

- Alternator output indication

- Manifold pressure indication

- Magneto drop (RPM variation)

- Electronic Ignition operation

- Throttle and mixture operation.  Lubricate as needed

- Engine idle speed – 650 – 700 RPM

- Full power static RPM at 2700

- Brakes operation

- Lights interior/exterior operation

- Oil/fuel leaks after shut down

- Aircraft documents

- Applicable Airworthiness Directives

- Instruments/ avionics operation, changes or repairs

Logbook Entry:

- Aircraft total time in service

- Name, signature, and certificate type and number of the person performing the inspection