## Pre-Start

Preflight 	complete

Seats	adjusted & locked

Seat belts	on

Passenger Brief	complete

Prop	full forward

Main Bus A & B	on

EFIS Battery	on

EFIS Boot	wait

## Starting Engine Cold

Throttle	open ½”

Mixture	full rich

Fuel Pump	on (count to 10)

Mixture	idle cut-off

Prop	clear

Magneto	on

Electronic Ignition	on

Starter	engage

Engine Start	mixture full rich

## Starting Engine Hot

Throttle	open ½”

Fuel Pump	on

Mixture	idle cut-off

Prop	clear

Magneto	on

Electronic Ignition	on

Starter	engage

Engine Start	mixture full rich

## Starting Engine Flooded

Throttle	full

Fuel Pump	off

Mixture	idle cut-off

Prop	clear

Magneto	on

Electronic Ignition	on

Starter	engage

Engine Start	mixture full rich

## After Start

Engine RPM	1000

Fuel Pump	off

Alternator	on

Volts & Amps	confirm

Oil Pressure	+60

Mixture	lean as required

Nav Lights	on

EFIS	set GPS source

GPS	enter flight plan

Comm Frequency	set to ATIS/ASOS

Fuel Quantity	set

Oil Service Time	check

## Before Takeoff

Flight Controls	free & correct

Engine RPM	1800

Mag Check	confirm 6 EGT

Electronic Ignition	confirm 6 EGT

Engine Instruments	green

Prop Exercise	RPM drop

Engine RPM	1000

Altimiter	set

Autopilot	set

Horizon	level

Magnetic Heading	reasonable

Trim	takeoff

Fuel Pump	on

Strobes	on

Mixture	set

Prop	forward

Comm Frequencies	set

Doors	closed & latched

Annunciators	fuel pump & speed brake ok

## Pre-Landing (G.U.M.Ps)

Fuel Pump	on

Gear	down & cured

Mixture	set

Prop	2700

Landing Light	on

## Instrument Approach Setup	(30+ miles out)

Get Destination Wx	runway/approach

Altimeter	set

Autopilot Source	GNS 430W

Approach	select & load IAF

Brief the Approach

Transition	IAF or Vectors

Inbound Course/Altitudes/Missed	brief

Tower/Unicom Frequency	set standby

VLOC Frequency	set standby

## Initiate Full Approach

Approach	activate

Nav Mode	GPS

Autopilot Mode	TRK

VLOC Frequency (ILS/LOC/VOR)	set

## Initiate Vectors to Final

Autopilot Mode	CRS

Autopilot Course	given vector

Approach	activate vector-to-final

GPS Navigation will SUSPend until aircraft is established on published leg.

## Cleared for the Approach

Navigation Active	verify SUSP off

Nav Source	set VLOC or GPS

Autopilot Mode	TRK

Needles	active

Comm Frequency	set

Landing Light	on

GUMPS	complete

Power Profile	2700-12-120

Runway Lighting	activate

Re-Brief the Approach

Inbound Course/Altitudes/Missed	brief

EFIS Missed Altitude	set

## Missed Approach

Enable Waypoint	 cancel SUSP

Engage Autopilot Climb	encoder

Nav Mode	 set GPS

Approach Descent Power Profile:

RPM	2700

MAP	12 inches

Airspeed	120

## Engine Shutdown

Mixture	idle cutoff

Overhead Switches	off

Electrical Busses	off

## V-Speeds

VNE	200	VSB 	110

VNO	170	VS	67

VA	140

VX	110	Best power off glide

VY	110	speed	110

## Engine Power Loss in Flight

Fuel Pump	on

Mixture	rich

Engine Gauges	check

## Power Off Landing

Locate suitable field

Establish spiral pattern 1000 ft. above field at downwind position for normal landing approach

When committed to landing:

Seat belt & harness	tight

Ignition	off

Main Bus A&B	off

Fuel Shutoff	off

Mixture	idle cutoff

## Holding Pattern

Inbound Leg

Holding Waypoint	selected and active

VLOC Frequency	set

Nav Mode	 VLOC or GPS

Select OBS Mode	 OBS

EFIS OBS Heading	set on HSI page

Autopilot	TRK

Crossing Waypoint

180 Outbound	or 3 seconds

Outbound	time 1 minute

180 Inbound	or 3 seconds

Inbound Turn Established

Autopilot	TRK

Established Inboud	time 1 minute

Go To Crossing Waypoint Step