Oil Change Checklist

| Supplies Needed |  |  |
| --- | --- | --- |
| Part | Qty | Part No. |
| Oil Filter | 1 | Tempest AA48110-2 Champion CH48110-1 |
| Oil | 10 Quarts | Phillips 66 X/C Aviation oil 20w50 |
| Camguard | 1 Pint |  |
| Oil Sample Test Kit | 1 |  |
| Copper Crush Washer 1.00 I.D. | 1 | MS35769-21 or AN900-16 |

- Remove cowl and check for wear and hinges for security

- Visually inspect engine mounts

- Warm engine to oil temp of 165 degrees

- During engine warm up, listen for any unusual anomalies and run a mag check

- Check braking action under load

- Inspect for visible signs of hard landings

- Drain the oil

- Take a mid-stream oil sample for an oil analysis

- Remove sump screen, inspect and clean

- Re-install sump screen with new crush gasket.  Torque to 300in/lbs and safety wire in place

- Remove oil filter, cut open and lay out media

- Pass a strong magnet over media looking for ferrous metal particles

- Install new oil filter, apply DC-4 or clean oil to the filter gasket, safety wire in place

- Visually inspect cylinders and engine for cracks, leaks, failed seals, etc.

- Visually inspect hoses, belts and engine accessories

- Visually inspect magneto exterior and spark plug leads

- Remove crankcase vent saddle.  Clean and decoke.  Reinstall and safety wire.

- Inspect engine baffles for wear, wrong orientation or cracks in metal components

- Visually inspect exhaust system for visible signs of leakage

- Physically check exhaust components for security

- Visually inspect prop and prop hub and spinner for wear, leaks, cracks or movement

- Check and adjust tire pressure

- Refill engine with oil & Camguard-- Fill to 6 quarts on dip stick.

- Insure aircraft is secure for a post oil change run-up

- Complete inspection for tools or any other foreign objects left in or around the engine

- Run up aircraft, check idle speed and idle mixture at shut down

- Check for leaks

- Re-cowl the engine and double check cowl security

- Clean all fingerprints and other signs of service from fuselage

- Reset engine timer in Stats Page of GRT EFIS.

- Make logbook entry stating service performed and parts and fluids added